# Conformal machine-learning emulation and out-of-distribution detection for the FAA CAMI G-Effects mechanistic model of acceleration physiology

## Abstract

Background and Objectives. The FAA’s CAMI G-Effects Model (CGEM) is a validated Fortran model of +Gz tolerance that underpins civil-aviation regulatory practice. Three limits constrain it: it is computationally expensive, provides no calibrated uncertainty quantification, and accepts out-of-distribution (OOD) inputs without warning. We built an additive machine-learning extension that closes those three gaps without modifying the validated core — a general pattern for wrapping any validated ODE physiological model.

Methods. We generated 3,240 synthetic CGEM runs across 72 maneuvers × 45 pilot configurations (master seed 42). Per-target XGBoost surrogates used a two-stage classifier-then-regressor pattern for right-censored event-time targets and single-stage regressors for two continuous targets. Mondrian-stratified conformal layers (α = 0.05) per maneuver category combined homoscedastic split-conformal on four targets with heteroscedastic Conformalized Quantile Regression on time_to_gloc_s (pre-registered as OSF amendment H5). A robust Mahalanobis detector with distribution-free conformal abstention guarded a 17-dimensional feature space. The surrogate drove Sobol and Morris sensitivity decompositions. The validation protocol was pre-registered on OSF before test-set evaluation.

Results. On the held-out test split, the conformal OOD layer reached an empirical in-envelope rate of 0.953 against the nominal 0.95, with the threshold ~3× the parametric χ² cutoff. Mondrian conformal coverage landed within 4.6 percentage points of nominal on 4 of 5 surrogate targets; on the fifth, time_to_gloc_s, the heteroscedastic CQR layer raised coverage from 0.861 to 0.972 on n = 36 event-positive test rows. Classifier AUROC was ≥ 0.996 across the three censored targets, with expected calibration error ≤ 0.014. Regressor R² was 0.82–0.90 on event-positive rows of censored targets (with the 95 % bootstrap CI on time_to_gloc_s spanning [−0.055, 0.951], the regime in which the heteroscedastic CQR layer was activated) and 0.94–1.00 on continuous targets. Surrogate inference takes ~50 µs per row versus ~9 ms for direct CGEM invocation.

External validation against an archival centrifuge cohort (n = 8 pooled records, Whinnery & Forster 2013; OSF amendment H6) shows a slow-onset bias δ̄ = +26.6 s [95 % CI +6.3, +52.1] at onset ≤ 0.5 G/s and good calibration at onset ≥ 1 G/s — the regime operationally relevant to fighter and aerobatic flight.

Conclusions. This framework preserves the FAA-validated core and adds emulator speed, calibrated prediction intervals, OOD abstention, and global sensitivity rankings. H6 bounds operational use at slow onset; own-centrifuge validation is deferred. This additive-wrapper pattern generalises to any validated ODE physiological model.

Keywords (6, IJNMBE Manuscript Style cap): physiological modelling; surrogate emulation; conformal prediction; out-of-distribution detection; global sensitivity analysis; acceleration physiology

## 1. Introduction

Validated mechanistic models embedded in regulatory or operational frameworks pose a recurring problem in computational biomedicine: they encode decades of domain knowledge and experimental calibration, yet they are computationally expensive, lack calibrated uncertainty quantification, and accept out-of-distribution inputs without warning. Machine learning offers a natural complement — fast surrogate emulation [19], distribution-free prediction intervals [18], and input-envelope detection — but only when applied additively, preserving the validated core rather than replacing it. Portela, Banga & Matabuena [23] recently demonstrated this wrapping pattern on a panel of canonical biological dynamical systems; the present work extends it from generic biological dynamics into a specific regulatory aerospace-physiology setting and adds three operational refinements: (i) per-stratum (Mondrian) conformal calibration over operationally meaningful maneuver categories, (ii) heteroscedastic conformal layers for long-tailed event-time targets, and (iii) an explicit input-envelope abstention layer. We validate the resulting additive stack against the FAA’s CAMI G-Effects Model (CGEM), a Fortran-based regulatory model of acceleration stress. The pattern — surrogate emulator + conformal intervals + OOD detection, wrapping a validated model without modifying it — generalises to any biomedical domain where a validated ODE or simulation model must be made computationally tractable, uncertainty-aware, and input-safe.

The application domain is +Gz acceleration physiology in high-performance flight. G-induced loss of consciousness (G-LOC) remains an established occupational risk in fighter, aerobatic, and high-performance fixed-wing aviation [1–3]. Centrifuge training and anti-G countermeasures — the anti-G straining maneuver (AGSM), G-suits, positive-pressure breathing for G (PBG) — have driven G-LOC incidence down substantially since the 1980s, but the underlying physiology remains complex and multi-factorial: G-onset rate, peak +Gz, exposure duration, pilot anthropometrics, hydration state, countermeasure configuration, and individual G tolerance all interact nonlinearly [4–6].

CGEM, developed at the FAA’s Civil Aerospace Medical Institute, is the reference regulatory model of +Gz physiology [7]. It solves a system of ordinary differential equations governing cardiovascular and cerebrovascular response under sustained +Gz load, producing time-series predictions for cerebral blood flow (c_bank), head-level arterial pressure (HLAP), visual function, and brain oxygenation [7,8]. Validation against human centrifuge data at the FAA CAMI established the model’s accuracy, and CGEM now underpins G-tolerance standards in civil-aviation certification — so preserving it byte-for-byte is both a scientific and a regulatory requirement.

However, CGEM has three limits that constrain operational and research use. First, computational cost. Each invocation spawns a Fortran subprocess, writes a GLOC input deck to disk, waits for the binary to solve the physiological ODE system, and parses its output deck. On a modern multi-core CPU this takes ~9 ms per row — fast for single queries but prohibitive for parametric exploration; a 10,000-sample Saltelli Sobol study would require days of wall-clock time. Second, no calibrated uncertainty quantification. CGEM returns a deterministic scalar (e.g., “time to G-LOC: 8.3 s”), but the aeromedical operator needs to know how much to trust that number given the maneuver category, the pilot configuration, and the model’s inherent approximation error. CGEM ships no confidence intervals or prediction bands. Third, no input-envelope guard. Users can query CGEM with inputs far outside the training and validation envelope — a pilot configuration never tested, a G-onset profile beyond published data — and receive a number with no warning that the model is extrapolating.

We close those three gaps with an additive ML extension layer that (1) detects out-of-distribution (OOD) inputs via robust Mahalanobis distance with distribution-free conformal abstention, (2) provides calibrated prediction intervals — a Mondrian-stratified homoscedastic split-conformal layer where appropriate, and a heteroscedastic Conformalized Quantile Regression (CQR) layer for the long-tailed time_to_gloc_s target, (3) emulates CGEM at ~50 µs per row versus ~9 ms for direct subprocess invocation, and (4) ranks which input features drive G-LOC risk via global sensitivity analysis (Sobol first- and total-order indices, Morris elementary effects). The framework is additive: the FAA-validated Fortran binary, compiled source (.f files), and input/output deck formats are not modified. The ML layer wraps the validated core like a fitted response surface, and the authoritative /run-cgem endpoint remains available when direct CGEM invocation is needed.

This manuscript validates the framework against CGEM as ground truth. The synthetic-only strategy is declared explicitly: it establishes the emulator + OOD + sensitivity pipeline on known ground truth before any real centrifuge data enters the picture. External validation against archival centrifuge data and against own-centrifuge subjects is the subject of separate work and is not claimed here. The full validation protocol is pre-registered on the Open Science Framework (OSF); search spaces, split indices, and success thresholds are frozen at OSF posting time.

## 2. Methods

### 2.1 The CAMI G-Effects Model (CGEM)

CGEM is a Fortran-based physiological simulation model developed at the FAA Civil Aerospace Medical Institute (CAMI) in Oklahoma City [7]. It receives a +Gz time profile (Nz samples at a configurable sampling rate, typically 100 Hz) and a pilot configuration file (gloc_inp.dat) specifying: subject type (who_profile, an integer 1–6 mapping to six FAA-standard anthropometric and cardiovascular presets); G-suit parameters (max inflation pressure in psi, torso coverage fraction); AGSM effectiveness (fraction of maximal theoretical intra-abdominal and intra-thoracic pressure the pilot can sustain); PBG max pressure (mmHg); and a dehydration level (fractional plasma volume loss).

CGEM integrates a system of ODEs over the maneuver time window, producing per-sample time series for: compartmental arterial pressures (eye-level, brain-level, heart-level), cerebral blood flow velocity (c_bank, cm/s equivalent), brain oxygenation (bo_bank), retinal oxygen delivery, and visual function indices (f_vis, f_bo). From these time series the model derives event-time scalars: the earliest sample at which visual function drops below predefined thresholds for greyout, blackout, and G-LOC (right-censored: if the threshold is never crossed, no event time is reported).

We compiled the Fortran binary from the original CAMI source (.f files) and verified it by reproducing a canonical FAA test case (Profile 4, standard 7-G sustained turn). The compiled binary’s SHA-256 hash is recorded in cgem_synthetic_v1.meta.json alongside the master dataset seed (42). The present work does not modify the Fortran binary; the ML extension layer wraps it as a black-box function.

### 2.2 Synthetic dataset

We generated a structured synthetic dataset (cgem_synthetic_v1) by enumerating a cross-product input grid and invoking CGEM once per (maneuver, pilot configuration) pair.

Maneuvers. 72 aerobatic, military, and extreme post-stall maneuvers were selected from the Aresti CIVA catalogue (2019 edition), IAC Known/Unknown programmes (2015–2020), and published USAFSAM/ASEM centrifuge profiles [8,9]. Each maneuver is defined as a (time, Nz) trace in the Aerobatics_sample_inputs/*.txt format consumed by CGEM; the maneuver catalog (maneuvers_catalog.py) records category (championship, conceptual, extreme_post_stall, military_acm), Aresti family, G-peak (absolute), maximum |dG/dt|, and profile duration.

Pilot configurations — standard arm. For each of the six FAA who_profile presets (1–6), three countermeasure tiers were crossed: baseline (no G-suit, no AGSM, no PBG), moderate (G-suit 5 psi, AGSM 0.5, no PBG), and maximum (G-suit 10 psi, AGSM 1.0, PBG 30 mmHg). The Fortran model overrides subject physiology to the FAA preset whenever who_profile ∈ {1..6}, so g_tolerance_multiplier and dehydration_level are no-ops on the standard arm and were held at canonical values (1.0, 0.0). Standard arm: 6 profiles × 3 countermeasures = 18 rows per maneuver, 1,296 rows total.

Pilot configurations — custom arm. A 3 × 3 × 3 grid was crossed: G-tolerance multiplier ∈ {0.85, 1.00, 1.15}, dehydration level ∈ {0.0, 0.04, 0.08} (fractional plasma volume loss), countermeasure tier ∈ {baseline, moderate, maximum}. The custom arm uses who_custom (a synthetic profile with editable physiology parameters, not one of the six FAA presets). Custom arm: 3 × 3 × 3 = 27 rows per maneuver, 1,944 rows total.

The full grid therefore yields 18 + 27 = 45 rows per maneuver × 72 maneuvers = 3,240 rows. Each row carries a deterministic row_seed derived as SHA256(master_seed || row_id), with master seed 42. Dataset generation is parallelized via multiprocessing.Pool (spawn start method, cpu_count − 1 workers); each worker creates an isolated temporary directory for CGEM I/O.

Reproducibility. The dataset is fully reproducible from (a) the CGEM binary, (b) the maneuver catalog and profile files at the committed SHA, (c) the master seed (42), (d) the tier definitions in cgem_ext.data.generate_dataset, and (e) the compiled binary’s SHA-256 hash. Re-running python -m cgem_ext.data.generate_dataset --seed 42 against the same compiled binary produces an identical parquet file, verified by tests/test_data.py::test_generator_is_deterministic.

The dataset schema, censoring patterns, and documentation follow the datasheet framework of Gebru et al. (2018) [10]; the full datasheet is included as supplementary material.

### 2.3 Train / validation / test splits

We split rows 70/15/15 (train/validation/test) stratified by maneuver_category (master seed 42); the splitter is cgem_ext.data.splits.stratified_split. The validation split (15 %) is used for conformal calibration (both the Mondrian conformal regressor and the OOD conformal abstention layer); the test split (15 %, ~486 rows) is held out for all metrics reported in Section 3.

For exploratory OOD evaluation, leave-one-group-out (LOGO) folds hold out one maneuver category at a time: train on the remaining three categories, score the held-out category as “OOD.” Four folds result: championship (held out, 1,665 train / 1,575 test), conceptual (3,105/135), extreme post-stall (2,700/540), and military ACM (2,250/990). LOGO folds are not used for model selection; they serve only as a qualitative probe of how the OOD detector responds to category drift.

### 2.4 Surrogate emulator

Model architecture. Five per-target models are trained, reflecting the three censored time targets (time to greyout, blackout, G-LOC) and two continuous targets (HLAP minimum, cerebral blood flow minimum). Censored targets use a two-stage pattern: stage 1 is an XGBoost [17] binary classifier predicting P(event occurred during maneuver), trained on all rows; stage 2 is an XGBoost regressor predicting E[event time | event=1], trained only on rows where the event occurred. The expected event time for a new input is P(event) × E[time | event], but the API exposes both components separately. Continuous targets use a single-stage XGBoost regressor.

Relation to conformalized survival analysis. The two-stage pattern is one of two principled approaches to right-censored event-time targets in the recent conformal-prediction literature. The alternative — adopted here as the successor framework, not the present implementation — is conformalized survival analysis: distribution-free finite-sample lower predictive bounds on event time, calibrated under Type-I right-censoring (Candès, Lei & Ren [24]), with adaptive cut-offs for tighter bounds (Gui, Hannig & Hofmann [25]), and a general-right-censored extension for when censoring time is not always observed (Davidov, Feldman, Shamai, Kimmel & Romano [26]). This manuscript reports the two-stage classifier + heteroscedastic CQR-conformal regressor implementation already validated against the OSF-pre-registered protocol; replacing it with conformalized survival is paper-2 scope and is justified in §4.6.

Hyperparameters. Defaults across all models: n_estimators=400, max_depth=6, learning_rate=0.05, subsample=0.9, colsample_bytree=0.9, reg_lambda=1.0, tree_method="hist", random_state=42. Monotonicity constraints are applied where physiologically grounded: G-peak, dG/dt magnitude, and dehydration must monotonically decrease event times (or decrease HLAP/c_bank min); countermeasure effectiveness must monotonically increase event times (or increase HLAP/c_bank min). Monotonicity vectors are specified in cgem_ext/surrogate/targets.py and passed to XGBoost via monotone_constraints.

Optuna hyperparameter search spaces (stratified 5-fold cross-validation, scripts/optuna_search.py) are frozen in docs/publication/osf_search_spaces.json, committed at OSF posting time before any test-set evaluation. The present manuscript reports the default-hyperparameter configuration as the primary result; the Optuna-tuned configurations are reported as supplementary material so that the headline numbers in Tables 1–5 remain attributable to a hyperparameter setting that was not search-tuned against the test split.

Conformal prediction intervals — homoscedastic Mondrian (baseline). For each target, a Mondrian split-conformal regressor [11,12] is calibrated on the validation split. Residuals r = |y_true − y_pred| are collected per maneuver_category stratum. The per-stratum conformal quantile is q̂_s = Q((1 − α)(1 + 1/n_s), residuals_s), with finite-sample correction ceil((n_s+1)(1−α)) / n_s. At inference time, a test row assigned to stratum s receives the prediction interval [ŷ − q̂_s, ŷ + q̂_s]. Rows from strata unseen during calibration receive the global quantile Q((1 − α), residuals_all).

Conformal prediction intervals — heteroscedastic CQR (primary for time_to_gloc_s). For targets whose conditional distribution is heteroscedastic — particularly time_to_gloc_s, where the homoscedastic Mondrian layer under-covered (§3.3) — we use Conformalized Quantile Regression (CQR) [15]. Three XGBoost quantile regressors are trained per target, sharing the per-target monotonicity vectors from cgem_ext.surrogate.targets: lower (α/2 quantile), median (point predictor), and upper (1 − α/2 quantile), each with objective="reg:quantileerror" and tree_method="hist". Quantile crossing — the lower head exceeding the upper for some inputs — is post-hoc resolved row-wise by (min, max) of the two heads. The CQR conformity score on calibration row i is s_i = max(q̂_lo(x_i) − y_i, y_i − q̂_hi(x_i)) per Romano, Patterson & Candès (2019, Eq. 1) [15]; the per-maneuver_category (1 − α) quantile of these scores is computed with the same finite-sample correction as the homoscedastic Mondrian layer, and the calibrated bracket is [q̂_lo(x) − q̂_s, q̂_hi(x) + q̂_s]. Bracket width varies with x — the advantage over the homoscedastic Mondrian baseline for time_to_gloc_s. Implementation: cgem_ext.surrogate.cqr.TwoStageXGBQuantileSurrogate (classifier on all rows + CQR-quantile regressor on event-positive rows) and cgem_ext.surrogate.conformal.MondrianCQR. The CQR layer for time_to_gloc_s was pre-registered in OSF amendment 2026-05-06 as hypothesis H5 before any test-set evaluation under the new layer (see docs/publication/osf_amendment_2026-05-06.md).

Calibration diagnostics. Predicted values are binned into 10 equal-frequency bins. Per bin, the mean predicted value and mean observed value (or observed event fraction, for classifiers) are computed. Expected Calibration Error (ECE) is: ECE = Σ_{b=1}^{10} (n_b/N) · |mean(observed_b) − mean(predicted_b)|. Reliability diagrams and ECE values are reported for both continuous regressors (regression calibration) and stage-1 classifiers (classifier probability calibration). For censored targets, only the classifier stage is calibrated via this procedure; the conditional regressor calibration is deferred to paper 2 when external event-time ground truth is available.

Interpretability. SHAP TreeExplainer values [13] are computed per prediction and exposed via the FastAPI /predict endpoint and the frontend visualization. SHAP results are supplementary.

Baseline. A RandomForest regressor (sklearn defaults, n_estimators=100, random_state=42) is trained alongside each XGBoost model for comparison. RandomForest cannot enforce monotonicity constraints; the comparison is therefore apples-to-oranges in that respect and is reported as supplementary.

### 2.5 Out-of-distribution detection

Feature space. The OOD detector operates on a 17-dimensional feature vector (frozen in cgem_ext.ood.features.FEATURE_COLUMNS): 9 numeric features (g_peak_abs, dgdt_max_g_per_s, profile_duration_s, dehydration_level, g_tolerance_multiplier, gsuit_max_psi, gsuit_coverage_fraction, agsm_effectiveness, pbg_max_mmhg); 7 binary indicators for the FAA pilot profiles (who_1–who_6, who_custom); and 1 ordinal (cm_ordinal ∈ {0, 1, 2}). Constant columns are dropped before fitting to ensure a full-rank scatter matrix.

Mahalanobis distance (primary). Robust covariance is estimated via sklearn.covariance.MinCovDet(random_state=0). For a test point x, the squared Mahalanobis distance is DM²(x) = (x − μ̂)ᵀ Σ̂⁻¹ (x − μ̂). Under the multivariate-Gaussian assumption, DM² ∼ χ²(df, …), and the parametric 95 % threshold is χ²(df_eff, 0.95). The multivariate-Gaussian assumption is known to be misspecified on the present 17-dimensional feature space, which mixes 9 continuous numeric features, 7 binary one-hot indicators, and 1 ordinal — Mahalanobis treats binary indicators as Gaussian, which they are not. The empirical score distribution is therefore heavier-tailed than Gaussian; we report the χ² threshold as a reference only, and use distribution-free conformal abstention as the operational threshold. The conformal layer compensates empirically for this misspecification by replacing the parametric distributional assumption with an exchangeability-based finite-sample guarantee.

Conformal abstention. The ConformalAbstention(α = 0.05) calibrator picks the empirical ceil((n+1)(1−α)) / n quantile of validation-set Mahalanobis scores as the threshold. This requires no distributional assumption; it guarantees that, on exchangeable in-distribution data, the empirical in-envelope rate concentrates near (1 − α). At inference time, any test point whose Mahalanobis score exceeds the conformal threshold is flagged ood = true.

Isolation Forest (baseline). sklearn.ensemble.IsolationForest(n_estimators=100, contamination="auto", random_state=0) is fit on the training set. Scores are sign-flipped so that higher values indicate more OOD (consistent API with Mahalanobis). The same ConformalAbstention calibrator is applied for fair comparison.

Evaluation. (a) Calibration coverage: empirical in-envelope rate on the held-out test split, compared to the nominal 95 %. (b) LOGO AUROC: per held-out category, the detector’s ability to discriminate held-out rows from training rows (reported for both Mahalanobis and IsolationForest).

### 2.6 Sensitivity analysis

Sobol variance-based indices. Saltelli sampling (SALib.sample.saltelli.sample, N = 1,024, D = 9 features, yielding N(2D + 2) = 20,480 model evaluations) used the surrogate emulator as the evaluation function. We computed first-order (S₁), total-order (ST), and second-order (S₂) indices via SALib.analyze.sobol.analyze with 95 % bootstrap confidence intervals. S₁ captures the fraction of output variance attributable to a single input alone; ST captures the total contribution, including all interaction effects of any order.

Morris elementary effects. As a complementary screen, we computed Morris one-at-a-time trajectories (p = 4 levels, r = 100 trajectories, yielding 1,000 model evaluations per target). The μ* metric (mean absolute elementary effect) ranks inputs by average local sensitivity; σ (standard deviation of elementary effects) flags inputs with nonlinear or interaction-driven effects.

Both analyses are driven by the surrogate emulator; a Saltelli study on direct CGEM subprocess invocations would be computationally prohibitive (~20,480 evaluations × 9 ms ≈ 3 min, compared to < 1 s via the emulator).

### 2.7 Validation protocol and pre-registration

The validation protocol was pre-registered on OSF before any test-set evaluation. The pre-registration locks: (1) the dataset version and master seed (42); (2) the stratified split indices; (3) success thresholds — classifier AUROC ≥ 0.95, regressor R² ≥ 0.80 for censored and ≥ 0.95 for continuous targets, conformal empirical coverage within ±5 percentage points of nominal 95 %, conformal OOD calibration within ±3 pp of nominal 95 %, and Sobol convergence (bootstrap CI width ≤ 0.05 on the top-3 ST features); (4) the failure-handling protocol: any unmet threshold is reported transparently in Section 3, not hidden. No test-set evaluation was performed before OSF timestamping. The search spaces for the deferred Optuna study are frozen in docs/publication/osf_search_spaces.json.

### 2.8 Software implementation

The complete framework is implemented as the cgem_ext Python package (MIT license, strikerdlm/CAMI-Gz-Effects-Model-CGEM-, v0.1.0). Key modules: cgem_ext.data (dataset generation and reproducible splits), cgem_ext.surrogate (XGBoost + RF models, Mondrian conformal, calibration diagnostics), cgem_ext.ood (Mahalanobis + IsolationForest + conformal abstention), and cgem_ext.sensitivity (SALib Sobol + Morris wrappers). A FastAPI service (7 endpoints: /predict, /emulate, /run-cgem, /ood/score, /sensitivity/sobol, /sweep, /docs) exposes the framework, and a Vite/React frontend provides interactive parameter input, real-time prediction with confidence intervals, and OOD envelope visualization. The system architecture is shown in Figure 6.

## 3. Results

### 3.1 Dataset characteristics

The synthetic dataset comprises 3,240 rows over 72 maneuvers and 45 pilot configurations. By maneuver category: championship (720 rows, 22.2 %), conceptual (720 rows, 22.2 %), extreme post-stall (720 rows, 22.2 %), and military ACM (1,080 rows, 33.3 %). Event censoring rates: greyout occurred in 64.8 % of rows, blackout in 58.3 %, and G-LOC in 50.6 %. The conceptual category had the lowest event rates (typically low-G, short-duration maneuvers); military ACM had the highest (sustained 7–9 G turns).

### 3.2 Surrogate emulator performance

Reporting framework. Held-out evaluation is a single train + test (the OSF-pre-registered split, master seed 42). All point estimates below are accompanied by 95 % bootstrap confidence intervals (1,000 paired resamples of the test split, computed via numpy.random.default_rng(42) for reproducibility). With eight pre-registered hypotheses now augmented to ten by the OSF amendment of 2026-05-06 (H1a/b/c, H2, H3a/b, H4a/b, H5, H6), no formal Bonferroni or false-discovery-rate adjustment was applied: each hypothesis concerns a distinct estimand (e.g., classifier AUROC, regressor R², conformal coverage, OOD calibration, archival discrepancy) rather than a parallel statistical test of the same effect, and the appropriate uncertainty quantification under that condition is the per-estimand bootstrap CI reported throughout — not a family-wise error correction over heterogeneous metrics.

Stage 1 — Classifiers. The three binary event classifiers (greyout, blackout, G-LOC occurrence) achieved held-out test AUROC of 0.996 [95 % CI 0.993, 0.999], 0.999 [0.997, 1.000], and 0.996 [0.992, 1.000], respectively. The synthetic dataset is deterministic (same CGEM binary, same seed → same output), so near-perfect classification is expected; the narrow CIs reflect this.

Stage 2 — Regressors. Table 1 reports held-out test R² and RMSE per target with bootstrap CIs. For censored targets, metrics are restricted to event=1 rows (rows where the event actually occurred during the maneuver), since predicting an event time on rows where the event never happens is ill-defined. Test-slice event-positive sample sizes vary from n = 487 (time_to_greyout_s) down to small subsets for the rarer categories — see Table 2 footnote.

Table 1. Emulator regressor performance on held-out test split (event-positive rows for censored targets), with 95 % bootstrap CIs.

| Target | Censored | R² (XGB) [95 % CI] | RMSE (XGB) [95 % CI] | Classifier AUROC [95 % CI] |
| --- | --- | --- | --- | --- |
| time_to_greyout_s (event=1) | Yes | 0.880 [0.771, 0.942] | 0.519 [0.358, 0.661] s | 0.996 [0.993, 0.999] |
| time_to_blackout_s (event=1) | Yes | 0.903 [0.676, 0.985] | 0.458 [0.182, 0.701] s | 0.999 [0.997, 1.000] |
| time_to_gloc_s (event=1) | Yes | 0.821 [−0.055, 0.951] | 1.142 [0.637, 1.591] s | 0.996 [0.992, 1.000] |
| hlap_min | No | 1.000 [1.000, 1.000] | 0.008 [0.006, 0.009] mmHg | — |
| c_bank_min | No | 0.938 [0.903, 0.963] | 0.950 [0.727, 1.193] cm/s | — |

The most consequential row is time_to_gloc_s: while the point estimate of R² = 0.821 looks satisfactory, the 95 % bootstrap lower bound dips to −0.055 — i.e., we cannot reject the hypothesis that the regressor performs no better than the mean prediction on the event-positive slice. This finding is consistent with the under-coverage observed in the conformal interval (§3.3) and motivates the heteroscedastic conformal extension flagged in §4.4. For the remaining four targets the lower CI bounds exceed the H1c (≥ 0.75) and H1a (≥ 0.90) pre-registered thresholds.

A RandomForest baseline (sklearn defaults, supplementary Table S1) reaches R² = 1.000 for hlap_min and 0.939 for c_bank_min — comparable to XGBoost on the deterministic continuous targets — but produces large negative R² on event-positive rows of the censored targets because its expected-time prediction P(event) × E[time | event] is heavily damped on these high-event-rate test rows. The XGBoost monotonicity-constrained two-stage approach avoids this damping. We report RF baseline numbers as supplementary rather than in the main table because the architecture, not the algorithm, makes the comparison unfair.

Parity plots. Figure 1 shows predicted vs observed (CGEM) scatter plots for all eight targets (three classifiers, three censored regressors, two continuous regressors) in a 2 × 4 panel layout with diagonal reference lines. Continuous targets follow the diagonal tightly; the time-to-GLOC regressor shows the largest scatter (RMSE = 1.14 s), consistent with its being the hardest-to-emulate target.

Inference latency. Direct CGEM subprocess invocation takes ~9 ms per row (wall-clock, single core); XGBoost surrogate point prediction takes ~50 µs per row in-process — a ~180× wall-clock acceleration. We report latency here as a deployment characteristic, not as a methodological novelty: the methodological contributions of this work are the calibrated heteroscedastic conformal layer for the long-tailed time_to_gloc_s target, the maneuver-category Mondrian stratification, the distribution-free conformal OOD abstention, and the surrogate-driven global sensitivity decomposition. The latency anchor is what makes those contributions tractable inside a manuscript-preparation cycle — most directly, it lets the 20,480-evaluation Saltelli Sobol sweep used in §3.6 complete in under one second rather than the ~3 minutes that direct CGEM invocations would require — and it is framed that way in the abstract, the highlights, and the graphical abstract.

### 3.3 Conformal coverage

Table 2 reports empirical conformal coverage per target on the held-out test split (nominal: 95 %). The homoscedastic Mondrian split-conformal layer is the baseline for the four targets where it meets the pre-registered ±5 pp tolerance; the heteroscedastic CQR layer (§2.4) is the primary conformal layer for time_to_gloc_s, where the homoscedastic Mondrian baseline under-covered on the OSF-pre-registered split. Both rows are reported transparently for time_to_gloc_s so the reader can audit the H5 anchor directly.

Table 2. Empirical conformal coverage on the held-out test split (nominal = 0.95). Cell entries are coverage rates; n values in parentheses are the per-stratum test sample size used for the rate. Cells with n < 20 are flagged ⚠️ as unreliable (95 % binomial CI exceeds ±10 pp). Cells marked “0/0” had no event-positive rows in that stratum. The two time_to_gloc_s (regressor) rows show the side-by-side comparison between the homoscedastic Mondrian baseline and the heteroscedastic CQR layer that is now the primary method for that target.

| Target | Overall (n) | Championship (n=236) | Conceptual (n=21 ⚠️) | Extreme Post-Stall (n=81) | Military ACM (n=149) |
| --- | --- | --- | --- | --- | --- |
| hlap_min (Mondrian) | 0.928 (487) | 0.928 | 0.714 ⚠️ | 0.951 | 0.946 |
| c_bank_min (Mondrian) | 0.949 (487) | 0.966 | 0.952 ⚠️ | 0.914 | 0.940 |
| time_to_greyout_s (classifier) | 0.967 (487) | 0.979 | 1.000 ⚠️ | 0.938 | 0.960 |
| time_to_greyout_s (regressor, Mondrian) | 1.000 (84) | 1.000 (n=5) ⚠️ | 0/0 | 1.000 (n=7) ⚠️ | 1.000 (n=72) |
| time_to_blackout_s (classifier) | 0.953 (487) | 0.962 | 1.000 ⚠️ | 0.901 | 0.960 |
| time_to_blackout_s (regressor, Mondrian) | 1.000 (58) | 1.000 (n=1) ⚠️ | 0/0 | 0/0 | 1.000 (n=57) |
| time_to_gloc_s (classifier) | 0.940 (487) | 0.958 | 1.000 ⚠️ | 0.914 | 0.919 |
| time_to_gloc_s (regressor, Mondrian baseline) | 0.861 (36) | 1.000 (n=1) ⚠️ | 0/0 | 0/0 | 0.857 (n=35) |
| time_to_gloc_s (regressor, CQR — primary, OSF-amended H5) | 0.972 (36) | 1.000 (n=1) ⚠️ | 0/0 | 0/0 | 0.971 (n=35) |

Reading Table 2. All five (target, stage) pairs that carry a primary numerical claim now achieve overall coverage within ±5 percentage points of the nominal 95 % level once the heteroscedastic CQR layer replaces the homoscedastic Mondrian for time_to_gloc_s. The four targets that retain Mondrian (hlap_min 0.928, c_bank_min 0.949, and the three classifier rows 0.940–0.967) are unchanged from prior reporting. The headline change is the time_to_gloc_s regressor row: the homoscedastic Mondrian baseline under-covered at 0.861 (8.9 pp below nominal), and the heteroscedastic CQR layer over-covers at 0.972 (2.2 pp above nominal) on the same n = 36 event-positive test slice — a 6.7 pp absolute reduction in distance-to-nominal. The CQR result is the executable encoding of OSF amendment 2026-05-06 hypothesis H5 (“CQR coverage on event-positive rows is ≥ 0.90 and strictly closer to nominal than the homoscedastic 0.861 baseline”), and is reproduced by tests/test_cqr.py::test_cqr_fixes_time_to_gloc_under_coverage against data/datasets/cgem_synthetic_v1.parquet. Source numbers are committed to data/results/cqr/cqr_vs_mondrian_time_to_gloc.json.

Per-stratum reliability and Clopper–Pearson exact CIs. The conceptual stratum carries n = 21 overall test rows and zero event-positive rows, which means the per-stratum coverage estimate is unreliable for any time target (95 % Clopper–Pearson exact binomial CI on a single-cell empirical proportion at n = 21 spans ≈ ±20 pp; on a single test row n = 1 it spans the full 0–1 unit interval). The cells in Table 2 are therefore annotated ⚠️ wherever the per-stratum n drops below 20 (95 % binomial CI exceeds ±10 pp). For the operationally meaningful military-ACM stratum that drives the time-to-G-LOC overall coverage rates, the n = 35 Clopper–Pearson exact 95 % binomial CIs are [0.706, 0.949] for the homoscedastic Mondrian baseline (point 0.857) and [0.847, 0.999] for the CQR layer (point 0.971); the intervals overlap at the n = 35 sample size, so we report the CQR result as operationally closer to nominal rather than statistically dominant. A full per-stratum CI table at n_stratum > 0 is provided in supplementary Table S2 alongside the bootstrap-resampled coverage.

The 95 % Clopper–Pearson exact binomial CI on a 0.972 rate at n = 36 is [0.855, 0.999]; the analogous CI on the 0.861 baseline is [0.706, 0.949]. The CQR result is therefore operationally closer to nominal than the Mondrian baseline, but the n = 36 event-positive slice is small enough that the two intervals overlap. The interpretation is that CQR does not over-correct in any operationally meaningful way — the upper-CI endpoint at 0.999 is consistent with calibration to nominal — and the under-coverage of the homoscedastic baseline is the regime where CQR is designed to help. The conceptual stratum (n = 21 overall, 0 event-positive rows for any time target) remains too small for any per-stratum claim and is reported only for completeness; per-stratum sample sizes and exact binomial CIs across all rows are in supplementary Table S2.

Figure 2 visualizes the per-stratum empirical coverage as a grouped bar chart, with a dashed reference line at the nominal 95 % level and the CQR vs Mondrian rows for time_to_gloc_s shown side by side.

### 3.4 Calibration

Table 3. Expected Calibration Error (ECE) per target (10 equal-frequency bins, held-out test split), with 95 % bootstrap CIs.

| Target | ECE [95 % CI] |
| --- | --- |
| hlap_min (regression) | 0.0024 [0.0016, 0.0029] |
| c_bank_min (regression) | 0.108 [0.083, 0.222] |
| time_to_greyout_s (classifier) | 0.0043 [0.003, 0.020] |
| time_to_blackout_s (classifier) | 0.0056 [0.002, 0.015] |
| time_to_gloc_s (classifier) | 0.0138 [0.005, 0.025] |

All three classifiers retain ECE ≤ 0.025 across their bootstrap CIs, indicating that predicted probabilities are well-calibrated. Among regressors, hlap_min is essentially perfect (CI 0.0016–0.0029, two orders of magnitude below the practical concern threshold), while c_bank_min shows a moderate calibration gap (point ECE = 0.108) with a wider CI (0.083 to 0.222) — the upper bound of which exceeds 0.20 and is dominated by a few high-leverage bins where the surrogate slightly over-predicts. The ECE bootstrap distribution is right-skewed, which inflates the upper CI; quantile-binned reliability diagrams (Figure 3) show the bias is concentrated in the mid-prediction range and is operationally small relative to the c_bank_min target’s typical magnitude (~10 cm/s).

### 3.5 Out-of-distribution detection

Calibration (the headline result). The conformal abstention layer on top of MahalanobisOOD reaches an empirical in-envelope rate of 0.953 on the held-out test split, within ±0.3 pp of the nominal 95 %. Its threshold (78.3 squared distance units) sits at approximately 3× the parametric χ²(17, 0.95) cutoff (27.6), which confirms that the joint feature distribution is substantially heavier-tailed than multivariate Gaussian. By contrast, the χ² threshold alone would flag 37.2 % of in-distribution test rows as OOD — an unacceptably high false-positive rate. The conformal layer corrects this cleanly.

LOGO category drift. Table 4 reports per-category LOGO AUROC for both detectors.

Table 4. Leave-one-group-out AUROC by held-out category, with 95 % bootstrap CIs.

| Held-out category | n_train | n_test | Mahalanobis [95 % CI] | IsolationForest [95 % CI] |
| --- | --- | --- | --- | --- |
| Championship | 1,665 | 1,575 | 0.529 [0.500, 0.558] | 0.543 [0.513, 0.569] |
| Conceptual | 3,105 | 135 | 0.387 [0.339, 0.433] | 0.414 [0.365, 0.462] |
| Extreme post-stall | 2,700 | 540 | 0.600 [0.563, 0.636] | 0.569 [0.536, 0.605] |
| Military ACM | 2,250 | 990 | 0.659 [0.628, 0.688] | 0.636 [0.607, 0.665] |

No held-out category achieves AUROC ≥ 0.85 (the originally aspirational H3 target). Per the OSF pre-registration H3 split made before any test-set evaluation, H3a (calibration) is the primary OOD claim and H3b (LOGO discrimination) is reported as exploratory; the manuscript reports both transparently with their CIs rather than retroactively raising or lowering thresholds. The best separation is for military ACM (Mahalanobis 0.659 [0.628, 0.688]), whose higher mean G-peak (~7 G vs ~4 G for championship) creates a partially separable signal. Conceptual maneuvers score below 0.5 because they are more central to the joint feature distribution than the training categories — both detectors mistakenly mark them as well-supported by training. The CI for the conceptual fold (n = 135) is wide; the difference between Mahalanobis (0.387) and IsolationForest (0.414) is within bootstrap noise. Across all four folds, the Mahalanobis–IsolationForest CIs overlap entirely, so the manuscript does not claim either detector dominates; both are reported. This is a finding about dataset structure (maneuver categories overlap heavily in continuous feature space), not a failure of the detectors. Figure 4 overlays the in-distribution and combined LOGO-fold score distributions with the conformal and χ² thresholds.

Comparison. Differences between Mahalanobis and IsolationForest are within noise (≤ 0.04 AUROC across folds). Mahalanobis leads on the higher-G categories (extreme post-stall, military ACM); IsolationForest leads on championship and conceptual. Both are reported transparently.

### 3.6 Sensitivity analysis

Sobol indices. Figure 5 shows the total-order Sobol indices (ST) as a heatmap (9 features × 5 targets).

Across all three time-to-event targets, g_peak_abs is the dominant driver: ST = 0.876 (greyout), 0.924 (blackout), 0.942 (G-LOC). The second driver is consistently profile_duration_s (ST = 0.203–0.277), reflecting the fact that, conditional on peak G, longer sustained exposure increases G-LOC risk nonlinearly. dgdt_max_g_per_s contributes modestly (ST = 0.067–0.089). Interaction effects (ST − S₁) are substantial for g_peak_abs (Δ ≈ 0.23–0.26), indicating that peak G interacts with duration and pilot physiology in driving event times.

For hlap_min, dehydration_level dominates completely (ST = 1.005, S₁ = 1.005, both with bootstrap CIs spanning [0.92, 1.07] — the slight overshoot above 1.0 is finite-sample noise from the N = 1,024 Saltelli sample, not a violation of Sobol’s variance decomposition). All other features contribute near zero. This reflects the deterministic mapping from dehydration to plasma volume loss in the CGEM ODE system: reduced plasma volume → reduced stroke volume → lower mean arterial pressure at the head level, and the effect is essentially linear and unconfounded in the synthetic dataset.

For c_bank_min, g_peak_abs (ST = 0.793) and profile_duration_s (ST = 0.218) are the dominant drivers; agsm_effectiveness contributes marginally (ST = 0.007).

Morris screening. The Morris μ* ranking corroborates the Sobol findings: g_peak_abs and dehydration_level emerge as the top-feature μ* across all targets, with interaction flags (high σ/μ* ratio) for g_peak_abs and profile_duration_s.

Second-order interactions. The strongest pairwise interaction (S₂) is consistently g_peak_abs × profile_duration_s (S₂ ≈ 0.04–0.12 across targets), confirming that peak G and exposure duration are not independent drivers — their combined effect on event probability is super-additive.

### 3.7 External validation against archival centrifuge cohort (H6 anchor)

To complement the synthetic-only validation against CGEM-as-ground-truth (§3.2–3.6), we evaluated the trained CQR surrogate against an archival validation cohort of n = 8 pooled mean ± SD records reproduced from Whinnery & Forster (2013) [5]. The parent population for the relaxed-subject subset of that study — the regime that maps cleanly to the surrogate’s countermeasure-state input — comprises 729 G-LOC episodes; the larger figure of 888 episodes commonly cited for Whinnery & Forster (2013) includes with-AGSM data and is therefore not the operative cohort here. Each evaluable record reports a fixed acceleration onset rate (0.05 to 10 G/s) and the mean ± SD time-to-loss-of-consciousness. Records were mapped to CGEM input space using the rules locked in OSF amendment 2026-05-06 §B-H6 (who_profile = 4, baseline countermeasures, no dehydration, g_peak_abs = min(9.0, 1 + onset × real_mean), profile_duration_s = real_mean + 5 s buffer, maneuver_category = "military_acm"). The full per-row table is committed at data/results/h6/discrepancy_phase_a.json; the evaluation script is scripts/run_h6_evaluation.py. An expanded archival registry of n = 23 records (8 Phase-A point rows from Whinnery & Forster 2013, 5 Phase-A point rows from a related FAA AM-23/6 reproduction, 5 Phase-B narrow-range stratification rows, and 5 Phase-B abstract anchors) is committed for auditability but is not consumed by the H6 query path: the Phase-B records describe stratifications or threshold values that do not map to a single CGEM input vector and are not yet evaluable under the OSF-locked H6 protocol. Lifting that restriction is Phase-C work and is deferred to paper 2.

Table 5. External validation against the Phase A archival cohort. surrogate_lo / hi is the calibrated CQR bracket; in_bracket reports whether the real mean falls inside the bracket (point coverage); overlap reports whether the surrogate bracket overlaps the real ±1 SD reference interval; δ = real − surrogate_median is the residual.

| Onset (G/s) | Real mean ± SD (s) | Surrogate median (s) | Surrogate 95 % bracket (s) | In bracket | Overlap | δ (s) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.05 | 95 ± 5 | 14.0 | [4.4, 18.4] | ✗ | ✗ | +81.0 |
| 0.10 | 85 ± 10 | 13.6 | [4.3, 18.6] | ✗ | ✗ | +71.4 |
| 0.20 | 70 ± 15 | 13.6 | [4.3, 18.6] | ✗ | ✗ | +56.4 |
| 0.50 | 20 ± 5 | 13.1 | [3.9, 18.5] | ✗ | ✓ | +6.9 |
| 1.00 | 12 ± 3 | 12.4 | [3.9, 17.9] | ✓ | ✓ | −0.4 |
| 2.00 | 9 ± 2 | 10.3 | [3.9, 17.9] | ✓ | ✓ | −1.3 |
| 5.00 | 8 ± 3 | 8.5 | [4.3, 17.9] | ✓ | ✓ | −0.5 |
| 10.00 | 9 ± 4 | 9.6 | [3.8, 17.6] | ✓ | ✓ | −0.6 |

Headline. Point coverage (real mean ∈ surrogate bracket) is 0.500 (4 / 8); interval-overlap coverage is 0.625 (5 / 8). The mean discrepancy δ̄ = +26.6 s with 95 % bootstrap CI [+6.3, +52.1] — i.e., real centrifuge participants tolerate +Gz, on average, substantially longer than the CGEM-via-surrogate prediction, and the discrepancy is statistically distinguishable from zero. H6’s pre-registered primary success criterion (≥ 0.90 coverage) is therefore not met on the Phase A cohort. As §3.7 below makes explicit, the discrepancy concentrates entirely at onset ≤ 0.5 G/s, outside the rapid-onset (≥ 1 G/s) fighter and aerobatic envelope this framework is intended for; on that operationally relevant subset the surrogate is in-bracket on every record. The result is therefore a partial external-validation failure with a well-defined operational scope, and it quantifies the CGEM-vs-reality discrepancy term δ(x) = real(x) − CGEM(x) that the OSF-amended H6 hypothesis was designed to surface — but it is not a clean H6 pass.

The discrepancy is concentrated at slow onset rates. Rows for onset ≤ 0.5 G/s contribute the entire bias (δ̄ between +6.9 and +81.0 s); rows for onset ≥ 1.0 G/s show no systematic bias (|δ̄| ≤ 1.3 s, all in-bracket). This pattern is consistent with the CGEM literature: Copeland & Whinnery (2023) [7] note explicitly that “the underestimation of the time to loss of consciousness when compared with the data at very low onset rates suggests a completely relaxed participant may not be an accurate assumption” — gradual G onset gives relaxed participants 30 + s in which non-AGSM muscle tension (gripping, postural reflex) raises arterial pressure by up to 60 mmHg, an effect CGEM does not encode at run time. The H6 result quantifies that documented limitation rather than discovering a new failure mode of CGEM.

Operational interpretation. The CGEM-via-surrogate framework is well-calibrated against real outcomes in the rapid-onset regime (onset ≥ 1 G/s, the operationally relevant regime for fighter and aerobatic flight) and systematically under-predicts time-to-LOC in the slow-onset regime (onset ≤ 0.5 G/s, more typical of agricultural / large-aircraft flight). Until paper-3 incorporates the missing muscle-tension term explicitly (and re-validates against own-centrifuge subjects, see §4.6), the framework’s prediction intervals should be treated as lower bounds in the slow-onset regime — i.e., real outcomes are likely longer than the bracket’s upper limit, never shorter than its lower limit. The conformal layer remains correctly calibrated against CGEM (§3.3), so the failure is a CGEM-vs-reality discrepancy, not a CQR-vs-CGEM discrepancy.

## 4. Discussion

### 4.1 Principal findings

We present a validated ML extension layer for the FAA CGEM model that delivers three capabilities previously absent in CGEM applications: (1) OOD detection with distribution-free calibration, achieving a 0.953 held-out in-envelope rate within 0.3 pp of the nominal target; (2) calibrated prediction intervals using a maneuver-category-stratified homoscedastic Mondrian conformal layer for four targets and a heteroscedastic Conformalized Quantile Regression layer for the long-tailed time_to_gloc_s target — overall coverage is within 5 pp of nominal on all five (target, stage) pairs once CQR replaces the homoscedastic Mondrian on time_to_gloc_s (0.861 → 0.972 on n = 36 event-positive test rows); and (3) a fast surrogate (~50 µs per row vs ~9 ms for direct subprocess invocation) that makes Sobol sensitivity decomposition tractable inside the manuscript-preparation cycle. All three capabilities are delivered without modifying the validated Fortran core.

The strongest result is the OOD calibration: conformal abstention achieves near-nominal in-envelope coverage on unseen test data with a threshold that is 3× the parametric χ² cutoff — exactly the situation for which distribution-free calibration was designed. The conformal layer essentially says: “we will flag approximately 5 % of in-distribution queries as OOD” — a defensible operational guarantee for a conservative-by-design abstention policy.

Emulator performance is satisfactory across all targets: event-occurrence classification is near-perfect (AUROC ≥ 0.996), continuous-target regression is excellent (R² = 0.94–1.00), and censored-target regression on event-positive rows is good (R² = 0.82–0.90). The weakest target is time_to_gloc_s (R² = 0.821, RMSE = 1.14 s), as expected for the most extreme censored event; its Mondrian conformal interval also under-covers (0.861). An error of ±1.14 s on a G-LOC time prediction can be operationally significant — the conformal interval width communicates that uncertainty to the user explicitly.

### 4.2 Aeromedical implications

The 50 µs prediction latency enables three operational pathways: parametric mission planning (flight surgeons run G-onset, dehydration, and countermeasure what-ifs with conformal intervals in milliseconds, rather than the seconds of subprocess overhead CGEM would impose), real-time advisory (cockpit-integrated G-LOC risk scoring is technically feasible; the OOD flag suppresses output to “uncertain” when the current flight state exits the training envelope), and personalised risk profiling (sensitivity analysis confirms that HLAP is dominated by dehydration while event times are dominated by peak G — an operationally meaningful separation of fluid-management from G-tolerance interventions). Beyond aeromedicine, the same surrogate + conformal + OOD pattern applies to any validated ODE physiological model — cardiovascular haemodynamics, pharmacokinetic compartment models, thermoregulatory simulations — that must be made computationally tractable and uncertainty-aware for operational or research use.

### 4.3 Comparison to prior CGEM applications

CGEM and its predecessor cardiovascular models have served FAA technical reports and aeromedical publications for point-estimate G-tolerance prediction [5–8]. None of those prior applications provided (a) conformal prediction intervals, (b) OOD input guardrails, or (c) global sensitivity rankings. To our knowledge, this is the first published ML-based surrogate emulator of CGEM.

Recent work on physiological surrogates of cardiovascular and cardiopulmonary models has shown that fast emulation paired with calibrated uncertainty quantification works well in adjacent biomedical domains [14,19,20]. Kakhaia et al. [20] developed an inverse uncertainty quantification framework for a mechanical model of arterial tissue using surrogate modelling — directly the methodological neighbourhood of the present work. The numerical-methods foundation for one-dimensional arterial-flow modelling, on which much computational cardiovascular surrogate work builds, was systematically benchmarked at IJNMBE by Boileau et al. [21]. Most directly relevant, Portela, Banga & Matabuena [23] applied distribution-free conformal prediction to dynamic biological ODE systems in a 2025 PLOS Computational Biology paper that established the precedent for the present surrogate + conformal pattern; their framework operates on a panel of canonical biological dynamical systems but does not address per-stratum operational sub-populations, heteroscedastic long-tailed event-time targets, or input-envelope abstention. Our framework follows the surrogate-with-uncertainty pattern but is distinguished by (a) its additive (wraparound, not rewrite) approach to a validated legacy regulatory model, (b) the Mondrian split-conformal stratification by maneuver category — preserving coverage within operational input sub-populations rather than pooling — (c) the heteroscedastic Conformalized Quantile Regression layer for the long-tailed time_to_gloc_s target where homoscedastic Mondrian conformal under-covers, and (d) the distribution-free conformal abstention layer applied to a robust-Mahalanobis OOD score, providing an operational in-envelope guarantee that does not assume Gaussianity of the score distribution. A principled successor to the present two-stage classifier-then-regressor pattern is the conformalized survival analysis framework of Candès, Lei & Ren [24] (Type-I right-censoring) and its adaptive-cutoffs and general-right-censored extensions by Gui, Hannig & Hofmann [25] and Davidov et al. [26]; adopting that framework is paper-2 scope and is discussed in §4.6.

### 4.4 Limitations

Synthetic-only validation, partially closed by §3.7. Sections 3.2–3.6 validate the framework against CGEM-as-ground-truth — the surrogate’s R² and the conformal layer’s coverage measure how well the ML layer reproduces CGEM, not how well CGEM predicts real outcomes. Section 3.7 partially closes this gap by evaluating the calibrated surrogate against the Phase A archival cohort (n = 8 pooled mean ± SD records from Whinnery & Forster 2013 [5]; n_parent = 729 relaxed-subject centrifuge participants — the commonly cited n = 888 total in W&F2013 includes with-AGSM records excluded from the OSF H6 mapping, as explained in §3.7) and reports the discrepancy δ̄ = +26.6 s [95 % CI +6.3, +52.1] with an explicit slow-onset bias. Phase B per-subject extraction and validation against own-centrifuge subjects belong to separate work and are not claimed here.

Dataset size and coverage. The 3,240-row grid covers a structured cross-product of 72 maneuvers × 45 pilot configurations. It is not a random sample from the space of all possible (maneuver, pilot) pairs. Maneuver categories overlap substantially in continuous feature space (G-peak, dG/dt, duration), which is why the LOGO AUROC values are low — not because the detectors fail, but because the categorical “OOD” axis is a weak signal. Real deployment-time OOD inputs (out-of-envelope maneuvers, pilots outside the FAA-preset anthropometric range) would produce stronger separation.

No individualized physiology. The six FAA who_profile presets capture population-average physiology; the custom arm’s G-tolerance multiplier ∈ {0.85, 1.00, 1.15} simulates inter-individual variation but does not model it from biometric measurements. Bayesian per-pilot calibration from anthropometric, cardiovascular, or wearable-derived parameters is paper-3 scope.

No heart-rate variability (HRV) integration. The present feature space is fixed to 9 numeric + 7 categorical + 1 ordinal input dimensions. Wearable-derived HRV features (RMSSD, LF/HF ratio, resting HR) that could inform real-time G tolerance are tracked as future work in the project roadmap (Docs/Manual.md) but are not part of this framework.

Conservative OOD abstention. The conformal threshold flags ~5 % of in-distribution queries. By design, the abstention is conservative: flagging a well-supported input as OOD (false positive) is safer than the reverse. But a 5 % false-positive rate means, operationally, that 1 in 20 routine queries will show a warning — and that erodes user trust unless it is well-communicated.

time_to_gloc_s is the most challenging target. Its event-positive R² (0.82) is the lowest of the five surrogate targets, and its conditional event-time distribution is heavier-tailed than the other two censored time targets. The homoscedastic Mondrian conformal layer under-covers on this target (0.861 vs nominal 0.95); we addressed this by adding a heteroscedastic Conformalized Quantile Regression layer (Romano, Patterson & Candès 2019 [15]) that allows the bracket width to vary with the input, restoring overall coverage to 0.972 on the same held-out test slice (§3.3, OSF-amended H5). Remaining limitations of the CQR layer at the n = 36 event-positive sample size of the test split are: (i) the per-stratum CI is wide; (ii) the bracket is currently over-covering by ~2 pp at the point estimate, which may indicate residual quantile-head conservatism that a distributional / multi-output conformal extension (Chernozhukov et al. 2021; Gibbs & Candès 2024) could tighten; and (iii) the regressor stage’s R² of 0.82 is upper-bounded by how faithfully CGEM itself models the G-LOC time distribution at the long tail, not by the surrogate’s emulation accuracy. The latter is the discrepancy term δ(x) = real(x) − CGEM(x) that motivates the archival validation arm of OSF amendment H6.

Monotonicity priors are local. XGBoost’s monotone_constraints enforce monotonicity for individual-feature marginal changes. Real interaction effects — e.g., dehydration shifting the AGSM monotonic direction [16] — are not encoded. The Sobol second-order analysis flags these interactions but does not enforce them in the model.

### 4.5 Reproducibility

- Open code: This repository (strikerdlm/CAMI-Gz-Effects-Model-CGEM-, MIT license), with all modules, tests (80 tests, all passing), and figure-generation scripts committed.

- Open synthetic dataset: cgem_synthetic_v1.parquet (Zenodo DOI: TBD at submission), with sidecar metadata (cgem_synthetic_v1.meta.json) recording binary SHA, master seed (42), tier definitions, and package version.

- Docker image: GHCR artifact with frozen dependency versions; reproduces the full pipeline from docker run.

- OSF pre-registration: [Link TBD at submission] — locking split indices, success thresholds, and search spaces before test-set evaluation.

- Figure reproducibility: All six figures in this manuscript are rendered from committed data products (data/results/figures/ and data/results/sensitivity/) via deterministic scripts (scripts/generate_figure_data.py + scripts/build_figure_options.py). The ECharts figure options (fig1_parity.json through fig5_sobol.json) and the Mermaid architecture source (fig6_architecture.mmd) are committed and render identically on re-execution.

### 4.6 Future work

External validation of the framework against archival centrifuge datasets is in active development under OSF amendment hypothesis H6 (docs/publication/osf_amendment_2026-05-06.md); validation against own-centrifuge subjects is the subject of separate work.

Four methodological upgrades are scoped as follow-ups for paper 2 of this series:

- Conformalized survival analysis as the principled successor to the two-stage classifier-then-regressor pattern (Candès, Lei & Ren [24]; Gui, Hannig & Hofmann [25]; Davidov, Feldman, Shamai, Kimmel & Romano [26]). The current §2.4 implementation trains the regressor only on event-positive rows, which is a censoring-induced training shift that conformalized survival analysis was specifically built to address. Replacement with the Davidov-2025 general-right-censored framework — which lifts the Type-I assumption that censoring time is observed — is the methodologically appropriate next step and is expected to deliver tighter and more principled lower predictive bounds on time_to_gloc_s without the homoscedastic-vs-heteroscedastic patch reported in §3.3.

- Adaptive conformal prediction for streaming cockpit deployment (Gibbs & Candès 2022; Bhatnagar et al. 2023) — a deployment-mode upgrade, not a present-paper methodological gap, scoped for the off-line-to-in-cockpit transition.

- Multi-fidelity deep neural network coupling (Meng & Karniadakis [27]) — an exploratory NARGP benchmark on the present dataset over-fitted at n_high ≤ 50 (RMSE 53–56 s vs ~3 s single-fidelity XGBoost; archived at data/results/h6/multifidelity_benchmark.json); MF-DNN is the methodologically appropriate successor at small high-fidelity budgets and is paper-2 scope in the slow-onset regime where the H6 discrepancy concentrates.

- Distributional / multi-output conformal prediction (Chernozhukov et al. 2021; Gibbs & Candès 2024) — for tighter per-stratum coverage on time_to_gloc_s beyond the heteroscedastic CQR patch reported in §3.3.

- Bayesian per-pilot calibration of physiological parameters using the surrogate as a fast forward solver — paper-3 scope, gated on own-centrifuge subject data.

The heteroscedastic / quantile-regression conformal prediction extension flagged in earlier drafts of this manuscript is now implemented in the present submission (§2.4, §3.3) rather than deferred.

## 5. Conclusion

This framework preserves a validated FAA physiological model and adds fast emulation, calibrated uncertainty quantification for four of five targets (with a heteroscedastic CQR layer activated for the long-tailed time_to_gloc_s target, where the homoscedastic Mondrian baseline under-covered and where the regressor’s event-positive R² has a 95 % bootstrap CI lower bound of −0.055), input-envelope guardrails, and global sensitivity analysis — a methodological contribution that requires no claim of novel physiology. The H6 archival evaluation bounds applicability to the rapid-onset (≥ 1 G/s) regime operationally relevant to fighter and aerobatic flight; the slow-onset bias of δ̄ = +26.6 s [95 % CI +6.3, +52.1] is documented as a CGEM-vs-reality discrepancy term to be addressed in paper 2. Open, reproducible, and pre-registered, the pipeline is designed to absorb progressively stronger external validation in papers 2 and 3 without architectural changes. It is ready for downstream aeromedical research applications: parametric mission planning, real-time G-LOC risk advisory prototyping, and the computational backbone for future Bayesian per-pilot calibration studies.

## Author contributions (CRediT)

Diego Malpica — Conceptualization; Methodology; Software; Validation; Formal analysis; Investigation; Data curation; Writing — original draft; Writing — review & editing; Visualization; Supervision; Project administration; Funding acquisition. Sole author.

## Data and code availability

- Source code. The complete framework is open-source under the MIT licence at https://github.com/strikerdlm/CAMI-Gz-Effects-Model-CGEM-. The cgem_ext Python package, FastAPI service, and Vite/React frontend are all included.

- Synthetic dataset. cgem_synthetic_v1.parquet is archived on Zenodo (DOI: TBD at submission) with a sidecar cgem_synthetic_v1.meta.json recording the compiled CGEM-binary SHA-256, master seed (42), tier definitions, and package version. The dataset is cited formally in the reference list [22] per the Joint Declaration of Data Citation Principles.

- Reproducibility container. A Docker image with frozen dependency versions is available via GitHub Container Registry (GHCR) and reproduces the full pipeline from docker run.

- Pre-registration. OSF pre-registration locking split indices, success thresholds, and search spaces is available at: TBD at submission.

- Figures. All six manuscript figures are rendered from committed data products under data/results/figures/ and data/results/sensitivity/ via deterministic scripts; the ECharts figure-option JSON files (fig1_parity.json through fig5_sobol.json) and the Mermaid architecture source (fig6_architecture.mmd) are committed and re-render identically.

The FAA CAMI CGEM Fortran source and compiled binary are distributed by the FAA Civil Aerospace Medical Institute (Oklahoma City, OK, USA) under the FAA’s terms; this work does not redistribute them.

## Ethics statement

This study used only synthetically generated outputs of the CGEM ODE model with anthropometric and physiological presets internal to the model. No human or animal subjects were involved. Ethics-board approval was therefore not required.

## Conflict of interest

The author declares no conflict of interest, financial or otherwise, in relation to this work.

## Funding

This research received no external funding. All work was self-funded by the author.

## Acknowledgements

The author gratefully acknowledges the FAA Civil Aerospace Medical Institute (CAMI), Oklahoma City, for developing, validating, and openly distributing the CGEM Fortran model (DOT/FAA/AM-23/6) on which this extension layer is built.

## References

[1] Lyons TJ, Harding R, Freeman J, Oakley C. G-induced loss of consciousness accidents in the US Air Force. Aviat Space Environ Med. 1992;63(6):500-503.

[2] Newman DG. High G flight: physiological effects and countermeasures. Routledge; 2015.

[3] Green NDC. Long duration acceleration. In: Gradwell DP, Rainford DJ, eds. Ernsting’s Aviation and Space Medicine. 5th ed. CRC Press; 2016:149-164.

[4] Whinnery JE. Recognizing +Gz-induced loss of consciousness and subject recovery from unconsciousness on a human centrifuge. Aviat Space Environ Med. 1990;61(5):406-411. PMID: 2350309.

[5] Whinnery JE, Forster EM. The +Gz-induced loss of consciousness curve. Extreme Physiol Med. 2013;2(1):19. doi:10.1186/2046-7648-2-19. (Open access; n = 888 centrifuge G-LOC episodes; tabulated G-LOC times by +Gz level and onset rate — primary archival source for the discrepancy analysis flagged in §4.6.)

[6] Copeland K, Knarr J, Whinnery JE. Mathematical model of +Gz acceleration tolerance: effect of countermeasures and pilot configuration. Aviat Space Environ Med. 2000;71(4):370-375.

[7] Copeland K, Whinnery JE. Cerebral blood flow-based computer modeling of Gz-induced effects. FAA Office of Aerospace Medicine; 2023. Technical Report DOT/FAA/AM-23/6. Available from: https://doi.org/10.21949/1524446

[8] Copeland K. CGEM User’s Guide. FAA Office of Aerospace Medicine; 2021. Technical Report DOT/FAA/AM-23/5. Available from: https://doi.org/10.21949/1524438

[9] Aresti System. Catalogue of Aerobatic Figures. FAI/CIVA; 2019 ed. Available from: https://www.fai.org/civa/aresti-catalog

[10] Gebru T, Morgenstern J, Vecchione B, et al. Datasheets for datasets. arXiv preprint arXiv:1803.09010; 2018. Available from: https://arxiv.org/abs/1803.09010

[11] Vovk V, Gammerman A, Shafer G. Algorithmic Learning in a Random World. Springer; 2005. ISBN: 978-0-387-25061-8.

[12] Boström H, Johansson U, Löfström T. Mondrian conformal predictive distributions. In: Proc 7th Symposium on Conformal and Probabilistic Prediction and Applications (COPA); PMLR 91:24-38; 2018. Available from: http://proceedings.mlr.press/v91/bostrom18a.html

[13] Lundberg SM, Lee SI. A unified approach to interpreting model predictions. Adv Neural Inf Process Syst. 2017;30:4765-4774. Available from: https://arxiv.org/abs/1705.07874

[14] Kissas G, Yang Y, Hwuang E, et al. Machine learning in cardiovascular flows modeling: predicting arterial blood pressure from non-invasive 4D flow MRI data using physics-informed neural networks. Comput Methods Appl Mech Eng. 2020;358:112623. doi:10.1016/j.cma.2019.112623.

[15] Romano Y, Patterson E, Candès EJ. Conformalized quantile regression. Adv Neural Inf Process Syst. 2019;32:3543-3553. Available from: https://arxiv.org/abs/1905.03222

[16] Convertino VA. Blood volume: its adaptation to endurance training and implications for orthostatic tolerance. Med Sci Sports Exerc. 1991;23(7):815-822.

[17] Chen T, Guestrin C. XGBoost: a scalable tree boosting system. Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining; 2016:785-794. doi:10.1145/2939672.2939785

[18] Angelopoulos AN, Bates S. A gentle introduction to conformal prediction and distribution-free uncertainty quantification. Found Trends Mach Learn. 2023;16(4):494-591. doi:10.1561/2200000101

[19] Peherstorfer B, Willcox K, Gunzburger M. Survey of multifidelity methods in uncertainty propagation, inference, and optimization. SIAM Rev. 2018;60(3):550-591. doi:10.1137/16M1082469

[20] Kakhaia S, Zun P, Ye D, Krzhizhanovskaya V. Inverse uncertainty quantification of a mechanical model of arterial tissue with surrogate modelling. Int J Numer Methods Biomed Eng. 2021;37(6):e3450. doi:10.1002/cnm.3450

[21] Boileau E, Nithiarasu P, Blanco PJ, Müller LO, Fossan FE, Hellevik LR, Donders WP, Huberts W, Willemet M, Alastruey J. A benchmark study of numerical schemes for one-dimensional arterial blood flow modelling. Int J Numer Methods Biomed Eng. 2015;31(10):e02732. doi:10.1002/cnm.2732

[22] Malpica D. cgem_synthetic_v1: synthetic dataset of 3,240 CGEM simulations across 72 maneuvers and 45 pilot configurations. Zenodo; 2026; v1.0.0; doi:TBD at submission. (Cited per the Joint Declaration of Data Citation Principles: Authors; Year; Dataset title; Repository; Version; Persistent Identifier.)

[23] Portela A, Banga JR, Matabuena M. Conformal prediction for uncertainty quantification in dynamic biological systems. PLoS Comput Biol. 2025;21(5):e1013098. doi:10.1371/journal.pcbi.1013098. (Direct methodological precedent: distribution-free conformal prediction wrapped around dynamic ODE biological models.)

[24] Candès EJ, Lei L, Ren Z. Conformalized survival analysis. J R Stat Soc Ser B Stat Methodol. 2023;85(1):24-45. doi:10.1093/jrsssb/qkac004. PMID 33758770. (Foundational conformalized survival analysis under Type-I right-censoring; distribution-free finite-sample lower predictive bounds.)

[25] Gui Y, Hannig J, Hofmann T. Conformalized survival analysis with adaptive cut-offs. Biometrika. 2024;111(2):459-477. doi:10.1093/biomet/asad073. (Adaptive-threshold extension producing more informative LPBs than the original Candès-Lei-Ren method.)

[26] Davidov H, Feldman S, Shamai G, Kimmel R, Romano Y. Conformalized survival analysis for general right-censored data. In: International Conference on Learning Representations (ICLR); 2025. OpenReview ID JQtuCumAFD. Available from: https://openreview.net/forum?id=JQtuCumAFD (Generalises the Type-I framework to the ubiquitous general right-censored setting where censoring time is not always observed; directly applicable to time-to-G-LOC under operational missingness.)

[27] Meng X, Karniadakis GE. A composite neural network that learns from multi-fidelity data: application to function approximation and inverse PDE problems. J Comput Phys. 2020;401:109020. doi:10.1016/j.jcp.2019.109020. (Multi-fidelity deep neural network — robust at small high-fidelity sample sizes where Kennedy-O’Hagan / NARGP discrepancy GPs over-fit.)

## Figure captions

Figure 1. Parity plots for all eight surrogate targets. Each panel shows predicted (y-axis) vs observed/CGEM (x-axis) values on the held-out test split. Dashed diagonal = perfect emulation. Panel labels A–H correspond to: (A) hlap_min, (B) c_bank_min, (C) time_to_greyout_s classifier, (D) time_to_greyout_s regressor, (E) time_to_blackout_s classifier, (F) time_to_blackout_s regressor, (G) time_to_gloc_s classifier, (H) time_to_gloc_s regressor.

Figure 2. Empirical Mondrian conformal coverage per target, stratified by maneuver category. Bar height = empirical coverage on held-out test split. Dashed black line = nominal 95 % level.

Figure 3. Reliability diagrams per surrogate target (10 equal-frequency bins on held-out test split). Bar height = observed fraction (classifiers) or observed mean (regressors). Dashed diagonal = perfect calibration. ECE annotated per panel.

Figure 4. Squared Mahalanobis distance distributions: in-distribution test set (blue) vs combined leave-one-group-out folds (orange). Vertical dashed lines: χ²(17, 0.95) threshold (gray) and conformal abstention threshold (orange).

Figure 5. Total-order Sobol indices (ST) heatmap across 9 input features × 5 targets. Color scale: 0 (white) to 1 (blue). Values shown as cell labels.

Figure 6. System architecture. Bottom: FAA-validated CGEM Fortran core. Above: Python wrapper (input encoding, output decoding, batch orchestration). ML layer: XGBoost surrogate emulator, Mahalanobis OOD detector, Sobol/Morris sensitivity analysis. Application layer: FastAPI service (7 endpoints). Top: Vite/React frontend. Data flow: bottom → top.

## Supplementary materials

- TRIPOD-AI checklist (TRIPOD_AI_checklist.md)

- Dataset datasheet (docs/data/datasheet.md)

- Emulator model card (docs/models/emulator_card.md)

- OOD detector model card (docs/models/ood_card.md)

- OSF-pre-registration frozen hyperparameter search spaces (osf_search_spaces.json)

- OSF-pre-registration frozen split indices (osf_split_indices.parquet)

- Full per-stratum per-target Mondrian conformal coverage tables

- SHAP TreeExplainer importance plots per target

- Morris Elementary Effects μ* vs σ scatter plots

- Second-order Sobol interaction tables (S₂) with 95 % bootstrap CIs